# HOW TO GLAZE A PIECE OF FIRED CERAMIC

BEFORE MIXING

Open the glaze bucket or container and pour off excess water into a jug and keep to one side.

(you may need to add this all back in after testing for thickness)

MIX GLAZE LEFT IN BUCKET THOROUGHLY

With a fork or stick or your hand

TEST FOR THICKNESS

By dropping a fingerful of glaze onto the back of your object, or dip a similar piece of bisqueware (fired clay) into bucket.

GETTING THE THICKNESS CORRECT

Can be different for each glaze

clear/transparent = postage stamp thickness

check test tiles which are standardised with three thicknesses single, double and triple to see how thickness alters the finished look

IF TOO THICK

Add back water gradually, and test thickness each time until correct or required thickness is achieved.

APPLICATION

CAN BE BY BRUSHING, DIPPING OR POURING

ALWAYS MIX GLAZE BETWEEN APPLICATIONS

ASK YOUR TUTOR, TECHNICIAN OR FELLOW STUDENTS FOR A DEMONSTRATION